The unit can be updated through a USB drive as follows:

- Copy the update file “MERGE.bin” to the root directory of a USB drive.

- Connect the USB drive to the USB port on the rear of the device.

Attention: The USB port can only support maximum 500ma. Please use a small power U-disk as upgrading USB drive.

- Connect an HDMI display to DV-MFMV-74/DV-MFSS-71.

- Turn on DV-MFMV-74/DV-MFSS-71. The display device displays the HDMI output signal after normal boot of the device, displaying the FSR logo.  When FSR logo goes off, proceed to Step 5 below.

- Press and hold the INPUT 1 button for more than five seconds, "System is upgrading…"is displayed in the display device and all button indicators on the front panel blink.  The upgrade process takes a little more than a minute to complete.

Attention:  Do NOT power-off the unit until the upgrade progress is complete, otherwise the unit firmware will be corrupted and unit may have to be returned to factory.

- After updating, “Upgrading is successful, system will reboot” is displayed in the display device.  After a few seconds, the unit will reboot.

- The unit reboots automatically.

- This completes the MERGE.bin upgrade process.